# Understanding config example json

Beginner guide to pi/config.example.json

config.example.json shows the structure of the private config.json file that app.py reads at startup. The example contains placeholders, not real credentials. A real config.json must be created locally on each Pi and kept out of Git.

# The big idea

JSON is a small text file with named settings. This file identifies which caregiver API and paired device the Pi should use.

# What this file receives and produces

Receives: Project specific API address, Firebase web API key, unique device email and password, camera preference, rotation, AI model, and face threshold.

Produces: Values read by app.py, cloud.py, service.py, and recognition.py when the real config.json exists.

# Read the code from top to bottom

## apiBase

The caregiver website and API base address. cloud.py adds /api/ paths to it.

## firebaseApiKey

The Firebase web project key used by the sign-in request. This is not an Admin service account key and does not by itself grant caregiver access.

## deviceId and deviceEmail

The unique paired Pi identity. X-Device-ID is sent with the Firebase ID token, and the backend checks that the two belong together.

## devicePassword

The Firebase Auth password for this Pi’s device account. It is sensitive and belongs only in the private config.json.

## camera and rotate

camera can be auto or a camera address. rotate is the clockwise angle used to orient captured images.

## model and faceThreshold

model selects the scene AI model. faceThreshold controls the default SFace similarity cutoff. A different dlib threshold may be supplied separately.

# A concrete example

The example value YOUR_FIREBASE_WEB_API_KEY must be replaced in config.json with the actual project setting. Keep the example file unchanged for reference. A second Pi should use its own device ID, email, and password so commands do not mix.

# Words to know

Configuration: Values that change how one installation behaves.

Placeholder: Example text that must be replaced before use.

Credential: A secret or identity value used for sign-in.

Boolean: A true or false value, although this particular example mainly uses strings and numbers.

# Check your understanding

Question: Should config.json be pushed to GitHub?

Answer: No.

Question: Can two Pis safely share one device ID and password?

Answer: No. They would receive the same device commands and status could conflict.

Question: Does firebaseApiKey replace devicePassword?

Answer: No. Both are used in the sign-in request for different purposes.